GRIME AI

Feature Extraction Module — Validation Test Plan

University of Nebraska-Lincoln

# 1. Purpose

This test plan defines procedures, test images, expected values, and acceptance criteria for validating the GRIME AI feature extraction module. Validation is conducted using two approaches: analytical tests that verify computed metric values against mathematically derived expected values, and directional tests that verify expected relative behavior between pairs of synthetic images with known characteristics.

# 2. Scope

Validation covers the following feature extraction metrics:

- Intensity — mean grayscale pixel value

- Shannon Entropy — pixel intensity histogram entropy

- Green Chromatic Coordinate (GCC) — G / (R + G + B)

- Excess Green Index (ExG) — 2g - r - b (normalized)

- HSV — dominant hue, saturation, and value via K-means clustering

- GLCM — contrast, homogeneity, and energy via Gray-Level Co-occurrence Matrix

- Gabor — filter response energy at specified frequency and orientation

- Fourier — high-frequency energy content of the magnitude spectrum

# 3. Test Environment

| Software under test | GRIME AI — Feature Extraction module |
| --- | --- |
| Validation script | grime_ai_feature_validation.py |
| Generate test images | python grime_ai_feature_validation.py generate |
| Run validation | python grime_ai_feature_validation.py generate --validate |
| Image folder | Feature Extraction Test Images (same directory as script) |
| Image format | PNG — lossless, required for analytical tests |
| Image dimensions | 256 x 256 pixels |
| Output | CSV files in Feature Extraction Test Images/validation_results/ |

# 4. Test Image Description

Analytical test images are solid-color PNG images with exact, uniform pixel values. Directional test images include synthetic patterns designed to produce predictable relative behavior between metric values.

| Filename | Description | Used In |
| --- | --- | --- |
| pure_red.png | Solid red (R=255, G=0, B=0) | Analytical |
| pure_green.png | Solid green (R=0, G=255, B=0) | Analytical |
| pure_blue.png | Solid blue (R=0, G=0, B=255) | Analytical |
| pure_white.png | Solid white (R=255, G=255, B=255) | Analytical |
| pure_black.png | Solid black (R=0, G=0, B=0) | Analytical |
| mid_gray.png | Solid mid-gray (R=127, G=127, B=127) | Analytical |
| gradient.png | Horizontal gradient, black to white | Directional |
| horizontal_stripes.png | Horizontal black/white stripes, 8px wide | Directional |
| vertical_stripes.png | Vertical black/white stripes, 8px wide | Directional |
| checkerboard.png | 8x8 pixel black/white checkerboard | Directional |
| checkerboard_blurred.png | Checkerboard with Gaussian blur (kernel=31) | Directional |
| noise.png | Random pixel noise (seed=42) | Directional |
| uniform_gray.png | Solid uniform gray (R=128, G=128, B=128) | Directional |
| stripes_blurred.png | Horizontal stripes with Gaussian blur (kernel=61) | Directional |

# 5. Test Cases

Each test case includes three tables: Parameters (blue), Expected Results (orange), and Pass Criteria (green). The Result column in the Pass Criteria table is completed by the tester. Analytical tests specify exact expected values with tolerance. Directional tests specify the expected relative relationship between two images.

# 5A. Analytical Tests

Run the validation script and compare computed values against the analytically derived expected values below. All analytical test images are solid-color PNG files with uniform pixel values so expected outputs can be derived from first principles.

## Test 1 — Intensity

Validates that the mean grayscale intensity metric returns the correct value for solid-color images with known pixel intensities.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Input images | pure_white.png, pure_black.png, mid_gray.png |
| Metric | Intensity — mean grayscale pixel value via cv2.mean() |
| Tolerance | ± 0.01 |

Expected Results

| Image | R, G, B | Expected Intensity | Actual Intensity | Difference | Pass / Fail |
| --- | --- | --- | --- | --- | --- |
| pure_white.png | 255, 255, 255 | 255.00 |  |  |  |
| pure_black.png | 0, 0, 0 | 0.00 |  |  |  |
| mid_gray.png | 127, 127, 127 | 127.00 |  |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| pure_white.png Intensity = 255.00 ± 0.01 |  |
| pure_black.png Intensity = 0.00 ± 0.01 |  |
| mid_gray.png Intensity = 127.00 ± 0.01 |  |

## Test 2 — Shannon Entropy

Validates that Shannon entropy returns 0.0 for perfectly uniform images. A uniform image contains only one gray level, producing a degenerate histogram with a single non-zero bin. Entropy of a single-outcome distribution is mathematically 0.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Input images | pure_red.png, pure_green.png, pure_white.png, pure_black.png |
| Metric | Shannon Entropy — computed from pixel intensity histogram |
| Tolerance | ± 0.001 |

Expected Results

| Image | Description | Expected Entropy | Actual Entropy | Difference | Pass / Fail |
| --- | --- | --- | --- | --- | --- |
| pure_red.png | Uniform red — single gray level | 0.000 |  |  |  |
| pure_green.png | Uniform green — single gray level | 0.000 |  |  |  |
| pure_white.png | Uniform white — single gray level | 0.000 |  |  |  |
| pure_black.png | Uniform black — single gray level | 0.000 |  |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| All four images return Entropy = 0.000 ± 0.001 |  |

## Test 3 — Green Chromatic Coordinate (GCC)

Validates GCC = G / (R + G + B) against analytically computed expected values for pure-color images. For pure green, all channel sum equals G alone, so GCC = 1.0. For pure red or pure blue, G = 0, so GCC = 0.0. For pure white, all channels are equal, so GCC = 1/3.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Input images | pure_green.png, pure_red.png, pure_blue.png, pure_white.png |
| Metric | GCC = G / (R + G + B), computed per pixel, mean across ROI |
| Tolerance | ± 0.001 |

Expected Results

| Image | R, G, B | Formula | Expected GCC | Actual GCC | Pass / Fail |
| --- | --- | --- | --- | --- | --- |
| pure_green.png | 0, 255, 0 | 255 / (0+255+0) | 1.000 |  |  |
| pure_red.png | 255, 0, 0 | 0 / (255+0+0) | 0.000 |  |  |
| pure_blue.png | 0, 0, 255 | 0 / (0+0+255) | 0.000 |  |  |
| pure_white.png | 255, 255, 255 | 255 / (255+255+255) | 0.333 |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| pure_green.png GCC = 1.000 ± 0.001 |  |
| pure_red.png GCC = 0.000 ± 0.001 |  |
| pure_blue.png GCC = 0.000 ± 0.001 |  |
| pure_white.png GCC = 0.333 ± 0.001 |  |

## Test 4 — Excess Green Index (ExG)

Validates ExG = 2g - r - b using normalized channel values (r = R/(R+G+B), etc.). For pure green, normalized g=1, r=0, b=0, giving ExG = 2. For pure red, normalized r=1, g=0, b=0, giving ExG = -1. For pure white, all normalized values = 1/3, giving ExG = 0.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Input images | pure_green.png, pure_red.png, pure_white.png |
| Metric | ExG = 2g - r - b where r=R/(R+G+B), g=G/(R+G+B), b=B/(R+G+B) |
| Tolerance | ± 0.001 |

Expected Results

| Image | Normalized r, g, b | Formula | Expected ExG | Actual ExG | Pass / Fail |
| --- | --- | --- | --- | --- | --- |
| pure_green.png | 0, 1, 0 | 2(1) - 0 - 0 | 2.000 |  |  |
| pure_red.png | 1, 0, 0 | 2(0) - 1 - 0 | -1.000 |  |  |
| pure_white.png | 1/3, 1/3, 1/3 | 2(1/3)-1/3-1/3 | 0.000 |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| pure_green.png ExG = 2.000 ± 0.001 |  |
| pure_red.png ExG = -1.000 ± 0.001 |  |
| pure_white.png ExG = 0.000 ± 0.001 |  |

## Test 5 — HSV Dominant Color

Validates HSV hue, saturation, and value against known values for pure-color images. OpenCV computes hue in the range 0-180; GRIME AI scales by 2 to produce standard 0-360 degree hue values. Pure red has hue 0, pure green has hue 120 (60 × 2), pure blue has hue 240 (120 × 2). Saturated pure colors have S=255. Pure white has S=0 (achromatic).

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Input images | pure_red.png, pure_green.png, pure_blue.png, pure_white.png |
| Metric | HSV dominant cluster center — H scaled to 0-360, S and V in 0-255 |
| Tolerance | ± 1.0 for all values |

Expected Results

| Image | Expected H (0-360) | Expected S (0-255) | Expected V (0-255) | Pass / Fail |
| --- | --- | --- | --- | --- |
| pure_red.png | 0 | 255 | 255 |  |
| pure_green.png | 120 | 255 | 255 |  |
| pure_blue.png | 240 | 255 | 255 |  |
| pure_white.png | — | 0 | 255 |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| pure_red.png: H = 0 ± 1, S = 255 ± 1, V = 255 ± 1 |  |
| pure_green.png: H = 120 ± 1, S = 255 ± 1, V = 255 ± 1 |  |
| pure_blue.png: H = 240 ± 1, S = 255 ± 1, V = 255 ± 1 |  |
| pure_white.png: S = 0 ± 1, V = 255 ± 1 |  |

# 5B. Directional Tests

Directional tests verify that texture and complexity metrics change in the expected direction between pairs of synthetic images. Exact values cannot be computed analytically for these metrics, but the expected direction of change is well-defined. Each test reports actual values for both images and passes if the stated directional relationship holds.

## Test 6 — Shannon Entropy (Directional)

Validates directional behavior of Shannon entropy across image pairs with known complexity relationships. Images with more diverse pixel value distributions should produce higher entropy than images with fewer distinct values.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Image pairs | noise.png vs uniform_gray.png; gradient.png vs uniform_gray.png; gradient.png vs noise.png |
| Metric | Shannon Entropy — expected to increase with pixel distribution complexity |

Expected Results

| Image A | Image B | Expected Direction | Value A | Value B | Pass / Fail |
| --- | --- | --- | --- | --- | --- |
| noise.png | uniform_gray.png | Noise > Uniform |  |  |  |
| gradient.png | uniform_gray.png | Gradient > Uniform |  |  |  |
| gradient.png | noise.png | Gradient > Noise |  |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| Entropy(noise) > Entropy(uniform_gray) |  |
| Entropy(gradient) > Entropy(uniform_gray) |  |
| Entropy(gradient) > Entropy(noise) |  |

## Test 7 — GLCM Texture (Directional)

Validates directional behavior of GLCM contrast, homogeneity, and energy. A checkerboard pattern has maximum pixel transitions at distance=1, producing high contrast. A uniform image has no transitions, producing maximum homogeneity and energy. Random noise has high contrast but low homogeneity and energy.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Image pairs | checkerboard.png vs checkerboard_blurred.png; uniform_gray.png vs noise.png |
| Metric | GLCM at distance=1, angle=0 — contrast, homogeneity, energy |
| Library | scikit-image graycomatrix / graycoprops |

Expected Results

| Metric | Image A | Image B | Expected Direction | Value A | Value B | Pass / Fail |
| --- | --- | --- | --- | --- | --- | --- |
| Contrast | checkerboard.png | checkerboard_blurred.png | A > B |  |  |  |
| Homogeneity | uniform_gray.png | noise.png | A > B |  |  |  |
| Energy | uniform_gray.png | noise.png | A > B |  |  |  |
| Contrast | noise.png | uniform_gray.png | A > B |  |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| GLCM Contrast: checkerboard > checkerboard_blurred |  |
| GLCM Homogeneity: uniform_gray > noise |  |
| GLCM Energy: uniform_gray > noise |  |
| GLCM Contrast: noise > uniform_gray |  |

## Test 8 — Gabor Filter Response (Directional)

Validates that Gabor filters respond preferentially to oriented patterns. A Gabor filter oriented perpendicular to a stripe pattern should produce a stronger response than when oriented parallel to the stripes. Two filter orientations are tested: theta=pi/2 (responds to horizontal features) and theta=0 (responds to vertical features).

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Image pairs | horizontal_stripes.png vs vertical_stripes.png |
| Metric | Gabor filter mean energy magnitude at frequency=0.3 |
| Library | scikit-image gabor filter |

Expected Results

| Filter Orientation | Image A | Image B | Expected Direction | Value A | Value B | Pass / Fail |
| --- | --- | --- | --- | --- | --- | --- |
| theta = pi/2 (horizontal) | horizontal_stripes.png | vertical_stripes.png | A > B |  |  |  |
| theta = 0 (vertical) | vertical_stripes.png | horizontal_stripes.png | A > B |  |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| Gabor (theta=pi/2) response: horizontal_stripes > vertical_stripes |  |
| Gabor (theta=0) response: vertical_stripes > horizontal_stripes |  |

## Test 9 — Fourier High-Frequency Energy (Directional)

Validates that high-frequency energy in the Fourier magnitude spectrum is higher for images with sharp spatial transitions than for smooth or uniform images. High-frequency energy is measured as the mean magnitude in the outer 25% of the radial spectrum.

Parameters

| Script subcommand | python grime_ai_feature_validation.py generate --validate |
| --- | --- |
| Image pairs | checkerboard.png vs checkerboard_blurred.png; noise.png vs uniform_gray.png |
| Metric | Mean magnitude in outer 25% of Fourier radial spectrum |

Expected Results

| Image A | Image B | Expected Direction | Value A | Value B | Pass / Fail |
| --- | --- | --- | --- | --- | --- |
| checkerboard.png | checkerboard_blurred.png | A > B |  |  |  |
| noise.png | uniform_gray.png | A > B |  |  |  |

Pass Criteria

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| Fourier high-freq energy: checkerboard > checkerboard_blurred |  |
| Fourier high-freq energy: noise > uniform_gray |  |

# 6. Running the Validation Script

The validation script grime_ai_feature_validation.py automates image generation and local metric computation. Run it before or after running GRIME AI to produce reference values for comparison.

| Generate images only | python grime_ai_feature_validation.py generate |
| --- | --- |
| Generate and validate | python grime_ai_feature_validation.py generate --validate |
| Validate existing images | python grime_ai_feature_validation.py validate |
| Custom input/output folders | python grime_ai_feature_validation.py validate --input C:/images --output C:/results |
| Output — Analytical CSV | analytical_validation_YYYYMMDD_HHMMSS.csv |
| Output — Directional CSV | directional_validation_YYYYMMDD_HHMMSS.csv |

Note: The validation script replicates GRIME AI metric logic for reference comparison. GRIME AI is the authoritative implementation. For full validation, run the same images through GRIME AI and compare its CSV output against the script results.

# 7. Result Recording

Record the following for each test case after execution.

| Item | Description |
| --- | --- |
| Test ID | Test number (1 through 9) |
| Date / Time | Date and time the test was executed |
| GRIME AI Version | Software version under test |
| Script Version | Validation script filename and date |
| Analytical CSV | Filename of the analytical results CSV |
| Directional CSV | Filename of the directional results CSV |
| Tests Passed | Number of pass criteria met out of total for this test |
| Pass / Fail | Overall pass or fail for this test case |
| Discrepancies | Any metric where computed value differed from expected beyond tolerance, with actual values |

# 8. Overall Pass / Fail Criteria

The feature extraction module passes validation when all of the following conditions are met across all nine test cases.

| Pass Criterion | Result (Pass / Fail) |
| --- | --- |
| All analytical test metrics match expected values within stated tolerances |  |
| All directional test pairs produce the expected relative ordering of metric values |  |
| No metric returns NaN, infinity, or an error value for any synthetic test image |  |
| Script validation results are consistent with GRIME AI CSV output for the same images |  |

# 9. Notes

- All analytical test images must be saved as PNG. JPEG compression alters pure-color pixel values and will cause analytical tests to fail.

- The validation script requires scikit-image and pywavelets for GLCM, Gabor, and Fourier tests. Install with: pip install scikit-image pywavelets --break-system-packages

- GLCM, Gabor, and Fourier metrics are statistical descriptors without closed-form analytical ground truth. Directional tests are the appropriate validation approach for these metrics.

- HSV hue values are scaled by a factor of 2 in GRIME AI to convert from OpenCV's 0-180 range to the standard 0-360 degree range. Expected values in Test 5 reflect this scaling.

- Feature extraction from field imagery (real-world validation) is deferred to operational use and is not part of this synthetic validation plan.